“Program Logic Controllers”

- Program logic controller

  - Controllers – event driven controllers

Ex. Automatic drilling machine:

- control components: drilling machine / workpieces

- event procedure

Start lowering the drill

Start drilling

Stop drilling

Retract the drill

    - microprocessor controlled system

    - The programmable logic controllers - PLC

  - Hardware

-processor / CPU

-power supply unit

-programming device

-memory unit

-the input and output interface

-- discrete / digital / analog signal

-communication interface

  - Internal architecture

-. Bus : address, data, control, I/O system

1.3.1 The CPU

-Arithmetic and logic unit(ALU) :

+arithmetic : add, subtract

+logic: AND,OR,NOT, EXCLUSIVE-OR,..

1.3.2 The buses

The buses; the paths used for communication within the PLC

1.3.3 memory

-ROM , RAM, EPROM

- 1bits, 8-bit word , 16-bit word

- Memory size: 1K = 210=1024

Memory : 4K x 8  = 4 x 8 x1024 (bits)  = 4 x 1024 (bytes)

1.3.4 Input/output unit

1) For isolation, optoisolators(optocoupler)

  - PLC systems

- A single box ,modular/rack type

- Typical PLC: 6,8,12 or 24 inputs and 4,8, 16 outputs

- Mitsubishi Compact PLC

    - Programming PLCs

- Hand-held programming device /Desktop consoles/Personal computers

- Programming software: different from the manufacturers

-Mitsubishi(MELSOFT), Siemens(SIMATIC STEP 7)

-International IEC61131-3

2. Input-output devices

2.1 Sensor, transducers

1) types

- sensors: measure a specific physical terms

+ temperature , humidity, resistor,..

+ inertial sensors: IMU – accelerometer, gyroscope

-transducers : convert a physical form to a another form

+piezo(electrical   mechanical), motor,..

2)Performance

- Accuracy : +/- %, non-linearity , hysteresis error

-range(measurement)

-dynamics:

+ response time (time to 95% of the steady state with a step input)

+rise time: time from 10% to 90 or 95% of the steady state value

+settling time: time for the output to settle within 2% of the steady state value

-sensitivity:

+output change / to be measured input change

+ex. Thermocouple ( 20  μV/oC)

-stability / drift / zero drift

-repeatability

-reliability

What is different? Why?

We may call a pull-up resistor

2.1.1 Mechanical switches

2.1.1 limit switch

detect the presence or passage of a moving part

2.1.2 Proximity switch

2.1.3 Photoelectric sensors and switches

- transmissive/ reflective /

2.1.4 Encoders

2.1.5 temperature sensor

- Bimetallic

-Resistive temperature detector – Wheatstone bridge

-thermocouple : contact type

2.1.6 position / displacement sensor

- potentiometer

-LVDT(linear Voltage Differential Transformer)

-Capacitive type

2.1.7 Strain gauges

2.1.8 pressure gauge / pressure swith

2.1.9 Liquid level detector

2.1.10 Fluid flow measure

2.2 Output devices

2.2.1 Relay

2.2.2 Directional control valve

2.2.3 Motors

- armature / field coil / brush / commutator

- PWM(pulse width modulation)

Why is it needed?

- Brushless motor

2.2.4. Stepper motor

- Stepper motor: For each pulse input, it rotates each angle.

-direction : unipolar / bipolar

- Number systems

  - Binary system

-LSB / MSB

  - Octal and hexadecimal

- Octal : 3bits(0,1,2,…,7)

- Hexadecimal : 4 bits(0,1,2,…,9,A,B,C,D,E,F)

- BCD(binary code decimal) : 4bits

- Ladder and Functional Block Programming

Introduction

- S/W

+Machine code / Assembly language / High level language ( C, C++. Matlab, Ladder programming)

- Ladder programming : depends on the PLC manufactures

- International standard: IEC 1131-3(International Electrotechnical Committee)

+ Ladder programs(LAD)

+Instruction List(IL)

+ Sequential Function Charts(SFC)

+ Structures Text(ST)

+ Functional Block Diagram(FBD)

  - Ladder Diagram

  - Logic function

5.2.1 switch

- Standard IEC 1131-3 symbols

- Timing diagram

- Element name(variable) , and Address

5.2.2 AND

- Ladder and timing

5.2.3 OR

5.2.4 NOT

5.2.5 NAND

5.2.5 NOR

5.2.5 XOR

  - Latching

  - multiple output

  - Entering Program

  - EFunction Block

Example